# Edition 2 Video Script: How Each Robot Works

## Summary

In my opinion, this summary is enough to build the video from. If you need more detail or context, see the detailed section for each robot below.

SUMMIT XL, how a command works. Pressing the clicker tells the voice bridge on the laptop to start recording from the wireless mic. Pressing it again stops the recording, and Whisper transcribes it. The matcher then looks for one of the four stations in the text. If it finds one, the command goes to the ROS container, which sends it to the robot as a navigation goal, and the robot drives there. If it can't, the transcribed text goes to the LLM. If the LLM can map it to a station, it goes to the container the same way. If neither can, nothing is sent and the screen shows Not a valid command. Before leaving for any station, the robot waits 5 seconds so the clicker can be placed back on the tray.

SUMMIT XL payloads. The person uses a Logitech clicker and a Hollyland wireless mic, both carried on the robot's acrylic tray. A laptop with an RTX 4090 runs the voice software, the LLM and the ROS container. On the robot, the robot stack handles localisation, obstacle avoidance and driving.

Spot, how a command works. Voice commands work as on the SUMMIT: the clicker starts and stops recording, Whisper transcribes it on the NUC, and the parser looks for exactly one waypoint on Spot's map. If it finds one, Spot walks there. If not, the command is rejected and the monitor shows Not a valid command; there is no LLM on Spot. When Spot arrives at the handoff or storage, it tilts its body and reads QR codes through its front cameras. At the handoff it stores the package codes from the task card and walks to storage by itself. At storage it checks each box against that list and says whether it matched.

Spot payloads. The person uses the same clicker and wireless mic, carried on Spot's wooden tray. An Intel NUC runs the voice, QR and navigation software and drives a monitor that shows the status page. Spot's own body cameras handle localisation and read the QR codes. The Spot tablet stays with the overseer as the emergency stop.

Control panels. Each robot has a control panel in a second terminal for the overseer. On the SUMMIT and the Go2W, it can send a held goal early (1) or cancel it (2). On Spot, it can reset Spot to Ready, which levels its posture and stops it waiting for a QR code (1). It can also re-arm QR scanning (2) or cancel navigation (3). The space bar in the main terminal cancels a movement on every robot. None of these replace the robot's own emergency stop.

Go2W. Works like the SUMMIT, including the LLM fallback, with ROS 2 and Nav2 underneath. Its section lists the differences.

## SUMMIT XL: from "go to handoff" to arrival

The laptop does the listening and deciding; the robot does the localising, planning and driving. The two meet at one point: a goal pose sent over Wi-Fi.

Clicker

click: start / stop

Wireless mic

carries the voice

Status screen

shows what it heard

Voice bridge

records 16 kHz audio

Whisper large-v3

speech to text, under 0.5 s

Command matcher

finds the station name

Local LLM (gpt-oss)

reads unclear commands

Departure hold

5s to put the clicker on the tray

ROS container

looks up the map pose,

sends the goal

Lidar, IMU, odometry

measure room and motion

AMCL

matches scans to the map

move_base

plans route, sets speeds

Omni wheels

drive to the handoff

SUMMIT: the laptop listens and decides, the robot localizes and drives

What the person uses

Laptop (RTX 4090)

SUMMIT XL

unclear

goal sent over Wi-Fi

The matcher is the gate. A clear station name goes straight on to the departure hold; an unclear command goes to the LLM first. Nothing reaches the robot unless it ends up as exactly one station.

## SUMMIT XL: "go to handoff" step by step

From the second click to the wheels turning takes about half a second of computing; the rest is the 5-second loading countdown and the drive. Steps 1 to 7 run on the laptop, step 8 hands the command to the ROS container, and steps 9 and 10 happen on the robot.

| # | What happens | Part doing it | How it works | Status screen shows |
| --- | --- | --- | --- | --- |
| 1 | Robot waits | Laptop, voice bridge | Whisper large-v3 was loaded once at start-up (about 35 s) and stays in GPU memory | Ready |
| 2 | Person clicks once | Logitech clicker, USB receiver in the laptop | The bridge reads the button as a raw key event and takes exclusive hold of it, so it never types into other windows | Listening |
| 3 | Person says "go to handoff" | Hollyland wireless mic, receiver in the laptop | Audio is recorded at 16 kHz, mono; recording stops on the next click or after 20 s | Listening |
| 4 | Person clicks again | Clicker | Recording stops and is saved as a WAV file | Listening |
| 5 | Speech becomes text | Whisper large-v3 on the RTX 4090 | Forced to English, no randomness (temperature 0), primed with the station names; 0.16 to 0.47 s per command | Listening |
| 6 | Text becomes a destination | Command matcher, then the LLM if needed | Looks for a station name as whole words; exactly one station is accepted. If the command cannot be read cleanly, the LLM decides (see below) | Going to handoff, or Not a valid command |
| 7 | Loading time | Departure hold | Every goal is held for a 5 s countdown before the robot leaves, so the clicker can be placed on the tray; the overseer can send it early or cancel it from the control panel | Countdown ring, Going to handoff, and "Put the clicker on the tray" |
| 8 | Command enters the robot software | Docker container on the laptop (ROS Melodic) | The bridge runs the navigation assistant inside the container; it looks up the saved handoff pose. | Going to handoff |
| 9 | Goal is sent and the robot drives | ROS over Wi-Fi, robot navigation | The pose is published to the robot's move_base. The path is planned. The robot drives there, keeping track of where it is with the lidar. | Going to handoff |
| 10 | Robot arrives | Robot | The robot arrives and the bridge accepts clicks again | Ready |

While the robot drives: every click is ignored, whether it is a bump on the clicker in the tray or a deliberate press. This prevents duplicate goals, and it matters in a timed run: if a team sends the robot to the wrong station, the robot finishes that trip and has to be sent back, and the time lost counts against the team. The space bar on the laptop lets the overseer cancel the active movement; it is a software cancel, not an emergency stop. The robot's own e-stop remains the safety device.

The LLM (language model): a local model, gpt-oss:20b, runs on the laptop through Ollama. It is used only when the matcher cannot read a command cleanly. The model receives the command with a prompt that lists the four stations and must answer with a navigate command for exactly one of them, or reply Not a valid command. It cannot invent a destination.

Control panel: a second terminal on the laptop gives the overseer two keys while a goal is held. 1 sends the robot immediately, without waiting for the countdown. 2 cancels the goal and returns the screen to Ready; if the robot is already moving, 2 sends the same cancel as the space bar. The panel only talks to the voice bridge on the laptop, and it is not a safety control.

## SUMMIT XL payloads

- Logitech clicker: starts and stops recording; ignored while the robot moves or during the departure hold.

- Hollyland wireless microphone: carries the spoken command to the laptop.

- Lenovo Legion 9 laptop (RTX 4090): runs the voice bridge, Whisper, the status page, the control panel, the ROS container and the Ollama server for the LLM.

- Robot computer: runs the robot's localisation and navigation.

- Robot stack: the SUMMIT's own lidar, IMU, wheel encoders, camera and omni wheels, which it uses to localise, see obstacles and drive.

- Acrylic tray: carries the boxes, the clicker and the mic; it is filtered out of the lidar scan so the robot does not treat it as an obstacle.

## Spot: from "go to arm" to the QR check

The NUC listens, decides and reads QR codes through Spot's cameras; Spot itself walks its recorded map and keeps itself localised with its body cameras.

Clicker

click: start / stop

Wireless mic

carries the voice

Status monitor

Listening, QR stages

Voice feedback

codes stored, matched

Voice session

records, holds the lease

Whisper small

offline, on the processor

Command parser

one waypoint, else refuse

GraphNav client

sends the waypoint

QR monitor (OpenCV)

starts on arrival at

handoff and storage

Standing

stand re-sent every 3 s

Legs

walk the route

GraphNav map

named waypoints, route

Body cameras

localise, read QR codes

Spot: the NUC listens and reads QR codes, Spot walks its recorded map

What the person uses

NUC

Spot

Two arrows join the GraphNav client and the QR monitor. Arriving at the handoff or storage station starts a scan (down arrow). A task card stored at the handoff sends Spot on to storage without a voice command (up arrow); this is Spot's only automatic move.

## Spot step by step

Spot has two kinds of behaviour: a spoken command makes it walk, and arriving at the handoff or storage makes it scan a QR code on its own.

A. Spoken command: "go to arm"

| # | What happens | Part doing it | How it works | Status screen shows |
| --- | --- | --- | --- | --- |
| 1 | Spot stands ready | NUC, voice session | The session holds Spot's control lease and re-sends a stand command every 3 s, so Spot never sits between commands | Ready |
| 2 | Person clicks once | Logitech clicker, USB receiver in the NUC | Starts recording from the wireless microphone | Listening |
| 3 | Person says "go to arm" | Wireless mic on Spot's tray | Audio recorded at 16 kHz | Listening |
| 4 | Person clicks again | Whisper (small model) on the NUC processor | Transcribes offline; the model was downloaded in advance because Spot's network has no internet | Listening |
| 5 | Text becomes a destination | Command parser | Accepts natural phrasings ("go to the arm", "take me to the arm") but only if they name exactly one waypoint on the active map; vague, relative or chained commands are refused | Going to arm, or Not a valid command |
| 6 | Destination is found on the map | GraphNav map on Spot | The waypoint named "arm" is looked up in Spot's recorded map | Going to arm |
| 7 | Spot walks | GraphNav navigation | Spot goes to the found destination; the NUC checks progress until it reports goal reached | Going to arm |
| 8 | Spot arrives | NUC | Applies the posture set for that station (none at the arm) and returns to Ready. Arriving at the handoff or storage also starts a QR check (table B) | Ready |

B. Arrival QR check (handoff and storage)

| Moment | What Spot does | How it works | Status screen shows | Spot says |
| --- | --- | --- | --- | --- |
| Arrives at the handoff | Tilts its body 40 degrees upward so its front cameras face the card | Per-station pitch from the station configuration file | Waiting for QR code | (nothing) |
| Reads the task card | Stores one to three package codes | Front fisheye camera images every 0.5 s, decoded with OpenCV; only the PKG-A to PKG-E code set is accepted, and duplicates are kept | Color code stored, one package icon per code | How many codes it stored |
| Card stored | Walks to storage by itself | The handoff store is the only event allowed to start a movement; QR text is never treated as a command | Going to storage | (nothing) |
| At storage, shown a box label | Compares one code against the stored list | A match removes one entry from the list | QR code matched; one icon fades out | "QR code matched" |
| After a match | Levels its body and stands still for loading | Scanning stops until the next arrival | Ready | (nothing) |
| Shown a wrong box | Keeps looking | Nothing is removed; Spot stays tilted and scanning | QR code not matched | "QR code did not match" |

While Spot waits for a QR result or walks to a station, clicks are ignored, so nobody can send it away mid-check or give it a new command on the way. The space bar cancel is available here too, as on the SUMMIT XL. The voice feedback is pre-recorded audio played from the NUC's speaker output, timed to follow the screen.

Control panel: a second terminal on the NUC gives the overseer three keys. 1 returns Spot to Ready: it levels its body, stops waiting for a QR code and unlocks the clicker. 2 restores the station's tilt and re-arms QR scanning, for example to scan again. 3 cancels navigation. Like the SUMMIT's panel, it is not a safety control; the Spot tablet's emergency stop is.

## Spot payloads

- Logitech clicker: starts and stops recording; ignored while Spot walks or waits for a QR result.

- Hollyland wireless microphone: carries the spoken command to the NUC.

- Intel NUC 11: runs the voice session, Whisper, the command parser, the QR monitor, the voice feedback, the status page and the control panel; controls Spot through the Boston Dynamics SDK.

- Monitor: connected to the NUC; shows the status page, including the QR stages and one package icon per stored code.

- Spot's body cameras: keep Spot localised and see obstacles; the front fisheye cameras also read the QR codes.

- Wooden tray: carries one box per trip, plus the clicker and mic.

- Spot tablet: emergency stop and manual control, kept with the overseer during every run.

## Go2W (backup)

The Go2W works like the SUMMIT in every way the person sees: the same clicker, microphone, Whisper speech recognition, matcher with the LLM fallback (it runs on the same laptop that hosts Ollama), four stations, status screen and control panel with departure hold. The differences are underneath:

- Robot: a Unitree Go2W wheel-legged robot instead of a wheeled base.

- Navigation software: ROS 2 with Nav2 instead of ROS 1 with move_base.

- No Docker container: the voice tool sends the saved pose straight to Nav2's navigate-to-pose action.

- Stations are saved by clicking a 2D Goal Pose in RViz and copying the pose into a configuration file.

- Nav2 localisation and navigation run on the laptop; the robot's own bring-up starts when it boots.